ATC (Additional Terms and Conditions)

Technical Specifications which is mandatory

- Processor Architecture: Intel® C Series / AMD EPYC / Equivalent / Higher - Processor Make: AMD / Intel - Number of Cores per Processor: 16 or Higher - Processor Base Frequency (GHz): 2.8 GHz or Higher - Processor Generation: 5th Generation / Intel 5th Gen Xeon / AMD EPYC 9004/9005 Series or Higher - Cache(MB): 37.5 MB or Higher - Server Disk Bays: The server should be provided with minimum 16 × 2.5" Hot-Swap Drive Bays. - SSD Interface Types: - NVMe SSDs: U.2/U.3 NVMe PCIe Gen4/Gen5 (OEM standard for 2.5" NVMe drives) - SATA SSDs: Standard 6 Gbps/12 Gbps Enterprise SATA - NVMe SSD (2.5″ U.2/U.3 Enterprise NVMe SSDs): - Total Number of NVMe SSD Slots Available: 2 (minimum) - Number of NVMe SSD Slots Populated: 2 - Capacity Offered per NVMe SSD: 1.92 TB (1920 GB) - Type: 2.5" NVMe (U.2/U.3) Read-Intensive SSD, DWPD: 1 - Hot-Swappable: Yes (as supported by OEM) - Total NVMe Capacity: 2 × 1.92 TB = 3.84 TB - SATA SSD (2.5″ Read-Intensive SSDs): - Total Number of SATA SSD Slots Available: 8 (minimum) - Number of SATA SSD Slots Populated: 4 (minimum) - Capacity Offered per SATA SSD: 1.92 TB (1920 GB) - Type: 2.5" SATA Read-Intensive SSD, DWPD: 1 - Hot-Swappable: Yes - Total Capacity from SATA SSDs: 4 × 1.92 TB = 7.68 TB - Summary of Total Installed Storage Capacity: - Total NVMe SSD Capacity: 3.84 TB - Total SATA SSD Capacity: 7.68 TB - Combined Installed Storage: 11.52 TB - RAID Level Support: The server must support hardware RAID levels RAID 0, RAID 1, RAID 5, and RAID 10. - Number of Network Ports (LAN): The server must have a minimum of 4 Network Ports, configured as 2 × 10GBase-T (RJ-45) and 2 × 1GBase-T (RJ-45) or higher/equivalent configuration. - Monitor: A 27-inch IPS Monitor must be supplied with the following minimum specifications: Resolution: 1920 × 1080 (Full HD) or better, Display Type: LCD with LED backlight, Connectivity: HDMI Port - Keyboard and Mouse Set: A USB Keyboard and USB Mouse Set must be supplied (wired). - Software 1: Windows Server 2025 Standard Edition (64 bit) perpetual license should supply and must compatible with server specifications and also include a 3-user Remote Desktop User Cal’s License. The bidder must supply the license pack that covers total physical cores of the server and strictly supply the license accordingly. - Software 2: Microsoft SQL Server 2025 Standard Edition (64 bit) (Perpetual License) shall be supplied, and the software must be fully compatible with the server hardware and operating system specifications. - Software 3: A reputed and reliable Server Edition Compatible Antivirus Software must be supplied, ensuring total protection with 5 years’ validity and full compatibility with the server specifications. The server-version antivirus must support standard server environments and provide real-time protection against viruses, malware, ransomware, and other advanced threats, along with regular automatic updates. - Installation, Commissioning, Training Technical Support and Services: - Installation, Commissioning, Technical Support and Services for Servers and Software’s including Setup and necessary configuration for hosting of Website / Web Application / Web Portal along with databases including necessary configurations and enabling of features. - The Server should compatible with latest open source technologies related to websites / web applications / portals etc. along with compatible data bases. - All required Accessories like Server Rack Mounting Kit, Power Cables, Cable Manager etc. also included. - The Training must be provided for the installed applications and services. - Demonstration for efficient working of the whole system with appropriate bench marking test must be done. - On-Site Comprehensive OEM Warranty: 5 years 24 x 7 OEM support and Warranty. - OEM Compatibility: - All Servers must be from the same OEM and Must compatible with each other and also with other software and hardware specified in this tender. - The Components like Mouse, Keyboard and Monitor must be from the same OEM. - Benchmark: - SPEC CPU® 2017 Floating Point Rate (FP Rate): 480 or Higher / SQTC certification - SPEC CPU® 2017 Integer Rate (INT Rate): 340 or Higher / SQTC certification - SPEC CPU® 2017 Floating Point Speed (FP Speed): 248 or Higher / SQTC certification - SPEC CPU® 2017 Integer Speed (INT Speed): 130 or Higher / SQTC certification - OEM: - Tender-Specific Manufacturer Authorization Form (MAF): Wherever authorized bidders participate, a Tender-Specific Manufacturer’s Authorization Form (MAF) must be submitted with date, time, and full OEM details (name, designation, address, and official e-mail ID). The Institute reserves the right to verify the MAF with the issuing OEM at any stage of bid evaluation. - Service Center Availability: The bidder must have its own authorized service center to ensure prompt support and service to the end-user. - OEM Toll-Free Support: The OEM must provide a dedicated Toll-Free Telephone Number for service and technical support. - ISO 9001 Certification: The bidder or the OEM of the offered products must possess a valid ISO 9001 certification. - Product Datasheets: All quoted products must be accompanied by official, publicly available OEM datasheets for technical verification. In case of any unexplained discrepancy in technical parameters, the bid may be rejected. - Technical Support Escalation Matrix: A Technical Support / Service Escalation Matrix must be submitted by the bidder along with the bid. - Important Notice: It was noted that in case of any deviation or conflict between the GeM Specified Parameters and the Additional Terms & Conditions (ATC), the Additional Terms & Conditions (ATC) shall be considered as final and binding.

Delivery Site: Delivery F.O.R. CSIR Jadavpur Campus

2.       Installation and Commissioning: To be installed at site by the supplier within the delivery period. The supplier shall also have to impart necessary training for effective operation and maintenance of the goods.

3.        Warranty: 5 (Five) years from the date of successful installation and commissioning.

4.       Catalogue/Leaflet/Brochure: Catalogue / Leaflet / Brochure may be asked from the bidder to justify the specification/feature of their offered equipment.

5.       Certificate/Declaration required:

I. Make In India: The Bidder must furnish a “Make In India” declaration/Certificate issued by the Manufacturer of the offered product. Such Declaration should indicate about the class/category (Class I- or II) in which the offered product/s fall as well as the percentage of Local Input of the offered product and the detailed address of the place (manufacturing unit) where the input is being added The Declaration/Certificate should have full name and designation of the signatory along with his/her email ID, direct phone number, office address. MII declaration, if not issued by the manufacturer or incomplete in nature, shall not be considered as an eligible MII Document and in such event the bid shall be treated as non-responsive.

II.    MSE: The bidder who is the manufacturer of the offered item may submit valid documents for claiming them as eligible MSEs.

III.   Land Border Sharing Certificate [GFR 144 (xi)]: Certificate/declaration reg. Land Border Sharing is compulsory. Any bidder from a country which shares a land border with India will be eligible to bid in this tender only if the bidder is registered with the Competent Authority. While participating in bid, Bidder has to undertake compliance of this, failing which the bid shall be treated as non-responsive. Any false declaration and non-compliance of this guidelines, if found at a later stage, would be a ground for immediate termination of the contract and further legal action in accordance with the laws.

IV.  OEM Certificate (applicable for manufacturer only): In case the Bidder is the OEM, a declaration to this effect must be attached along with the offer. This certificate with valid date must contain Name, Designation, Email Address and Mobile Phone Number of the authorized signatory along with the address of the office.

V.    MAF Certificate (applicable for dealer only): In case the Bidder is offering products on behalf of the OEM, a Tender/Bid specific Manufacturer’s Authorization Certificate (MAF) issued by the manufacturer in favour of the bidder is required to be attached along with the quotation. The certificate should contain Name, Designation, Email Address and Mobile Phone Number of the authorized signatory along with the address of the office. MAF Certificate, if not bid specific or not covering these points, shall not be considered as an eligible MAF document and in such event the bid shall be treated as non-responsive.

VI. CSIR Guidelines on Disposal of Rejected Goods Procured on GeM- No payment shall be made for rejected goods. After intimation ofo the rejection / part rejection by CSIR-IICB, the Seller shall be liable to remove /lift back rejected Goods within 10days of communication of rejection of GeM or otherwise without any extra charge / cost to CSIR-IICB failing which suitable ground rent / warehousing charges would be payable by the Seller to CSIR-IICB. If the seller fails to remove / lift back such rejected Goods within 45 days period, CSIR-IICB shall have the right to dispose off such rejected goods at the risk and cost of the seller in terms of CSIR guidelines for disposal of rejected goods procured on GeM.

VII.   Any other certificate required by the user (if any)